a coil will work, but  the Test is best run with about a 4in. - 6in. piece of tc wire across a build decks posts (see picture). In Escribe (mod tab), set Mod Resistance to 0.  In wire profile,   Power = 1    Temp = 0  /  Preheat temp = 1   preheat Punch = 1     Preheat Time Limit = 0.   Upload To Device...attach deck with coil or wire to mod...pulse (quick) fire to get mod to check cold ohms (if new coil question asked, answer yes)...run Atomizer Analyzer...allow wire to cool back to room temperature (extremely important) & subtract the Cold Ohms from the Raw Ohms...difference will be the Mod Resistance..