Assembly
Installing the Screen
[image: image1.png]


[image: image2.png]


[image: image3.png]


[image: image4.png]


The OLED screen mounts to the DNA 250 using an 8 pin ZIF socket and a flexible cable to allow for design flexibility. The cable can be bent or folded (once) but care should be taken to not apply tension or strain to the area where the cable attaches to the screen itself.

	
	

	Step 1: Locate the ZIF connector on the DNA 250
PCB
	Step 2: Carefully and gently lift the locking tab on
the rear of the connector to vertical


	
	

	Step 3: Fully insert the flexible cable into the front
of the socket with the contact side towards the

PCB
	Step 4: Close the locking tab and press until the
connector gently clicks. Remove the clear screen protector by pulling on the green tab.


�








�








�








�











